Step-by-Step Instructions for LQOC implementation on TCL

Step 1: Load open loop PRBS excitation data in file Prepare_PRBS_Data_for_SysId.m and run this file. This script file prepares perturbation data sets for model identification and validation. It also estimates operating steady state (Ys,Us) and measurement noise matrix R from the experimental data

Step 2: Now run Generate_BlackBox_SS_Model.m. This file will identify matrices of a discrete time state space model of the specified order and save them in TCL_BlackBox_OE_SS.mat

Step 3: Now open TCL_MechModel_Identification.m and load your experimental data file, and run this file. This program identifies 3 heat transfer related parameters of the mechanistic model for Temperature Control Lab system using the raw PRBD test experimental data set and the mechanistic model available in the literature and nonlinear optimization. It also estimates operating steady state (Ys,Us) and measurement noise matrix R from the experimental data. At the end of the identification exercise, a data structure named TCL is created and the results are saved into file TCL_MechModel_Parameters.mat

Step 4:  Run file TCL_MechModel_Linearization.m This file will linearize the mechanistic using parameters identified in the previous step and generate a discrete time local linear perturbation  model. The results are finally saved in  TCL_MechModel_Parameters.mat

Step 5:  Run Compare_Linear_Models.m. This file will compare balck-box model and linearized mechanistic models using identification and validation data sets. It also compared step responses of the identified black-box model and linearized mechanistic model

Step 6: Run TCL_LQOC_Simulation_Main.m This file will design a LQR controller either using the linear black-box model or using linearized mechanistic discrete time model. Make sure that the Wx and Wu are tuned such that simulated responses look acceptable. The designed controller parameters are saved in .mat file, which is used in experimental work. ‘

Step 7:  Run file test_LQOC.m to conduct experiment first using linear mechanistic model based LQOC and, if time permits, black-box model based LQOC on the TCL systems. Each experiment takes 30 minites and results are saved in the end is .mat file.